video_basics

Document History:

| S.No | Versio n No. | Author | Section(s) Affected | Change(s) made | Approved Date | Approved by |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.0 | Razeena Nilufar M. | All | Initial |  |  |

TABLE OF CONTENTS

1. FUNDAMENTALS OF DIGITAL VIDEO	2

1.1. Pixel	2

1.2. RESOLUTION	2

1.3. Frame Rate	3

1.4. Color Spaces	4

1.5. Bit DEPTH	5

1.6. Bit Rate	6

1.7. Chroma Subsampling	6

2. Digital Video Formats	7

3.Compression Techniques	10

3.1. SPATIAL (intraframe)	10

3.2. Temporal (Interframe)	10

4. INTERLACED AND PROGRESSIVE SCANNING	10

4.1. Interlaced Scanning	11

5. Video Interfaces	11

5.1. HDMI (High-Definition Multimedia Interface)	12

5.2. DisplayPort	13

5.3. CSI-2 (CAMERA SERIAL INTERFACE)	13

5.4. DSI (Display Serial Interface)	15

6. HIGH-SPEED TRANSMISSION CABLES	16

7. Serializer / Deserializer (SerDes)	17

7.1. Need for SerDes in video PROCESSING	17

7.2. WORKING OF SERDES	17

8. Electromagnetic Compatibility	18

9. References	20

DIGITAL VIDEO

Sequence of digital images called frames shown rapidly one after the other to create the illusion of motion.

# 1. FUNDAMENTALS OF DIGITAL VIDEO

## 1.1. Pixel

The smallest unit of a digital image. A video is composed of millions of pixels displayed in succession to create a moving image. Each pixel holds color information (RGB values).

## 1.2. RESOLUTION

The number of pixels in each dimension that a video contains. Common resolutions include:

| SD (Standard Definition) | 720 x 480 |
| --- | --- |
| HD (High Definition) | 1280 x 720 |
| Full HD | 1920 x 1080 |
| 4K UHD | 3840 x 2160 |
| 8K UHD | 7680 x 4320 |

Higher resolution = more detail = larger file sizes and higher transmission bandwidth requirements.

Real-Time Example: Netflix auto-adjusts resolution based on your internet speed: Slow speed → 480p

High speed → 1080p or 4K

[Higher resolution = better clarity but requires more bandwidth.]

## 1.3. Frame Rate

Number of frames displayed per second (fps). Standard fps include:

- 24 fps (movies)

- 30 fps (short videos)

- 60 fps (sports/gaming)

Higher frame rate = smoother motion = more data per second.

Example: 	
1080p at 30 fps = ~1.5 Gbps uncompressed

1080p at 60 fps = ~3.0 Gbps uncompressed 
- Low fps can cause blurry and jerky motion

## 1.4. Color Spaces

Represents how color information is stored and interpreted.

- RGB :  Red, Green, Blue

- YCbCr :

Y: Luminance (brightness)

U/V or Cb/Cr: Chrominance (color difference signals)

Real-Time Example: Blu-ray, MPEG video, and broadcast TV use YCbCr 4:2:0 color space for efficient compression. RGB is used for display panels and editing tools like Photoshop or Premiere Pro.

## 1.5. Bit DEPTH

Bit Depth referes to the number of bits per pixel to reperesent a specific colour. Every pixel in a digital image is a combination of three primary colours – Red, Green , Blue color channel .

3 bits (23)  =  8 colour tones

4 bits (24)   = 16 colour tones

8 bits (28)   = 256 colour tones

16 bits (216)   = 65,536 colour tones

24 bits (224)   = 16.7 million colour tones (most commonly used)

Effect of the parameters on Transmission

Higher pixel count, resolution, bit depth increases the number of data to be transmitted and thus requires higher bandwidth for transmission, large memory size for storage and processing.

- 4K (3840×2160) needs 4× the data bandwidth of Full HD (1920×1080).

- Going from 30fps to 60fps doubles the transmission bandwidth.

- YUV 4:2:0 reduces color info by 50–75% compared to RGB.

- 8-bit (256 levels) vs. 10-bit (1024 levels) per channel. 10-bit increases data by 25%.

- 4:2:0 cuts bandwidth to 50% compared to full 4:4:4 RGB.

## 1.6. Bit Rate

The total amount of data / information that has to be transmitted per second.

- Constant Bitrate

- Variable Bitrate

Bit Rate = Resolution (No of pixels) x Bit Depth (Data per pixel) x Frame 
Rate

## 1.7. Chroma Subsampling

Chroma Subsampling is a technique used in image and video compression to reduce the amount of color (chroma) data while retaining full brightness (luma) information. Since the human eye is more sensitive to brightness than color, this method allows significant data reduction with minimal perceived quality loss.

- Luma (Y): Represents brightness. Preserved at full resolution.

- Chroma (Cb, Cr): Represents color information (blue-difference and red-difference). Reduced through subsampling.

| Format | Description | Color Samples per 4 Luma |
| --- | --- | --- |
| 4:4:4 | No chroma subsampling (full color) | 4 (top) + 4 (bottom) |
| 4:2:2 | Half horizontal chroma resolution | 2 (top) + 2 (bottom) |
| 4:2:0 | Half horizontal & vertical resolution | 2 (top) + 0 (bottom) |

Picture Format Notation

# 2. Digital Video Formats

Format =  Codec ( Algorithm used to encode the video ) + Container (wrappers that stores video stream and related content )

Common Codecs : Compress and decompress video:

- H.264 (AVC): Industry standard, used in streaming

- H.265 (HEVC): Successor to H.264, better compression

- VP9: Open-source alternative to H.265

- AV1: New royalty-free format for streaming

Common container formats: Wraps video, audio, subtitles, and metadata.

- MP4 (MPEG-4): Most universal format, supports H.264/H.265

- MOV (Movie): Apple

- AVI (Audio video Interleave): Microsoft format

- MKV: Open source, supports multiple streams

- WEBM: Open source, Google

| Container Format | Common Codecs Supported | Real-Time Example | Pros | Cons |
| --- | --- | --- | --- | --- |
| MP4 (MPEG-4) | H.264, H.265 (HEVC), AAC | - YouTube streaming (H.264) - Default recording format on iPhone/Android | Widely supported, efficient, high quality in small size | Limited codec flexibility |
| MKV (Matroska) | H.264, H.265, VP9, AV1, AAC, FLAC | - Movie and TV shows with multiple languages/subtitles | Supports many codecs/subtitles/audio tracks | Not as universally supported as MP4 |
| AVI (Microsoft) | DivX, Xvid, MPEG-4 | - Legacy CCTV footage - Older camcorder recordings | Simple, compatible with older systems | Large file size, poor compression efficiency |
| MOV (Apple) | ProRes, H.264, AAC | - Used in Final Cut Pro, iMovie exports or previews | High quality, ideal for post-production | Larger file sizes, best on Apple platforms |
| WebM (Google) | VP8, VP9, Opus | - HTML5 web videos - Embedded videos on modern websites for fast loading | Open-source, good for browsers | Limited hardware playback support |

| Codec | Type | Key Features | Used By / Applications |
| --- | --- | --- | --- |
| H.264 (AVC) | Advanced Video Coding | - Most widely used - Good quality with decent compression | - YouTube - Zoom - Netflix - CCTV systems |
| H.265 (HEVC) | High Efficiency Video Coding | - Successor to H.264 - ~50% smaller file sizes at same quality - Higher decoding complexity | - 4K Streaming - Blu-ray Discs - Apple devices |
| VP8 / VP9 | Open-source (Google) | - Alternatives to H.264/H.265 - Used in WebM format - Efficient for web use | - YouTube - Google Meet |
| AV1 | Open-source (Alliance for Open Media) | - Newest, highly efficient codec - Better compression than H.265 - Limited hardware support (growing) | - Netflix (Premium) - YouTube (High-quality streaming) |

VIDEO STANDARDS

Television broadcast standards developed to define how video signals are encoded, transmitted, and displayed on TVs across different regions of the world.

- NTSC (National Television System Committee)

Developed in the U.S. in the 1940s and updated in the 1950s to support color TV. It matches the 60 Hz electrical system by running at 30 fps (29.97 fps) with 525 interlaced lines.

Used primarily in North America, Japan, and parts of South America.

- PAL (Phase Alternating Line)

Designed in Germany to address color issues in NTSC. PAL has 625 lines and a frame rate of 25 fps, aligning with 50 Hz electrical systems.

Used in Europe, Asia, and Africa.

- SECAM (Sequential Colour and Memory )

Developed in France, SECAM also uses 625 lines and 25 fps but transmits color information differently (sequentially) to improve long-distance signal quality.

Used in France, Eastern Europe, and parts of the Middle East and Africa.

# 3.Compression Techniques

## 3.1. SPATIAL (intraframe)

- Works within a single frame (like a JPEG image).

- Compresses redundant pixel data in areas with similar color or texture.

- Reduces file size by eliminating unnecessary details in static regions of a frame.

## 3.2. Temporal (Interframe)

- Works across multiple frames.

- Tracks change between consecutive frames and stores only the differences (motion).

- Uses motion estimation and compensation to avoid storing similar data repeatedly.

- Significantly reduces data in scenes with little movement.

# 4. INTERLACED AND PROGRESSIVE SCANNING

4.1. Interlaced Scanning

- Displays each frame in two passes: first the odd lines, then the even lines.

- Used in older TVs and broadcast systems (e.g., 1080i).

- Reduces bandwidth but can cause flickering or motion artifacts.

4.2. Progressive Scanning

- Displays the entire frame in one pass, line by line.

- Provides smoother, clearer motion and better image quality (e.g., 1080p).

- Preferred in modern displays, streaming, and digital media.

# 5. Video Interfaces

## 5.1. HDMI (High-Definition Multimedia Interface)

- Transmits uncompressed video + audio

- Versions:

  - HDMI 1.4: Supports 1080p

  - HDMI 2.0: Supports 4K at 60 fps

  - HDMI 2.1: Supports 8K and 4K at 120 fps

Key points in HDMI transmission:

- Signal Generation: The source device generates video (RGB/YUV pixel data) and audio signals.

- Encoding via TMDS:

Each 8-bit data is encoded into a 10-bit format to reduce electromagnetic interference and maintain signal integrity.

Three TMDS data channels carry Red, Green, and Blue pixel data respectively.

One TMDS clock channel synchronizes transmission.

- Differential Transmission:

Signals are transmitted as differential pairs over shielded wires to minimize noise.

- Display-Side Decoding:

The sink device decodes the TMDS signals and reconstructs the video and audio.

- Use Cases:

  - Home entertainment (TVs, set-top boxes)

  - Gaming consoles

  - Audio/video receivers

## 5.2. DisplayPort

DisplayPort is a high-performance digital display interface developed by VESA, designed to transmit high-resolution video and multi-channel audio from a source device to a display.

- Uses packet-based data transmission.

- It supports resolutions up to 8K and beyond, making it ideal for professional displays, gaming, and enterprise applications. .

- Digital display interface by VESA

- Higher bandwidth than HDMI

- Supports multi-monitor setups (daisy-chaining)

- Used in:

  - PC monitors

  - Graphics cards

## 5.3. CSI-2 (CAMERA SERIAL INTERFACE)

- MIPI standard for interfacing cameras with processors

- Low power, high-speed

Example: Camera in a Smartphone

While taking a picture:

1️⃣️ Image Sensor Captures Light

⬇

2️⃣ Converts Light to Pixel Data

• Format: RAW8 / RAW10 / RAW12 / RAW14 / RAW28

⬇

3️⃣ Pixels are Organized into Image Frames

⬇

4️⃣ CSI-2 Layer Prepares the Data

• Splits image into data packets

• Adds headers & footers (frame boundary, pixel format, etc.)

Packet Structure Example:(Long Packet for Image Data)

| Header (4 bytes) | Payload (pixel data) | Footer (2 bytes) |
| --- | --- | --- |
| Data Type, VC ID, Word Count, ECC | RAW pixel data | CRC-16 |

⬇

5️⃣ Data Transmission Begins

• Transmits serially over 1–4 (or up to 8) data lanes + 1 clock lane

• Uses D-PHY / C-PHY / A-PHY as physical layer

• Supports up to 32 Virtual Channels (multi-camera/multi-stream support)

⬇

6️⃣ Processor (AP/ISP) Receives Packets

• Decodes and reassembles image

• Performs enhancements (e.g., noise reduction, white balance)

• Stores or processes the final image

CSI-2 data usually travels over:

- D-PHY – common for short distances (e.g., inside phones)

- C-PHY – alternative PHY that transmits more bits per clock

- A-PHY – for long distances (up to 15 meters!), like in cars

Feature of CSI-2:

- Fast – supports high resolutions like 4K, 8K, or even higher.

- Low power – great for portable devices.

- Low interference (EMI) – important for clean signal transmission.

- Scalable – works for single or multiple cameras.

- Reliable – used in mission-critical systems like automotive.

- Used in:

  - Smartphones

  - Embedded vision systems

## 5.4. DSI (Display Serial Interface)

- MIPI standard for driving small displays

- Low power, optimized for mobile use

- Used in:

  - Mobile phones

  - Tablets

  - Wearables

|  | Primary Use | Max Resolution | Audio Support | Cable Length |
| --- | --- | --- | --- | --- |
| HDMI | Consumer electronics (TVs, consoles) | 4K to 8K (TVs, consoles) | Yes | 15 m (1080p), ~2 m (4K+) |
| DISPLAY PORT | PC monitors, docks | 4K to 8K (monitors, high-end GPUs) | Yes | 2 m for 4K+ |
| CSI 2 | Camera modules (mobile, embedded) | 1080p to 4K (camera sensors) | No | Short (≤30 cm) |
| DSI | Mobile displays (phones, tablets) | 1080p to 2K (mobile displays) | No | Short (≤30 cm) |

# 6. HIGH-SPEED TRANSMISSION CABLES

| Cable Type | Speed Support | Distance | EMI Resistance | Typical Usage |
| --- | --- | --- | --- | --- |
| Coaxial | Up to ~10 Gbps | Medium (~100m) | High | TV, Internet, RF, digital audio |
| STP | Up to 10 Gbps+ | Medium (~100m) | High | Ethernet, AV in EMI-prone environments |
| UTP | Up to 10 Gbps | Medium (~100m) | Low-Medium | Ethernet, phones |
| Fiber Optic | Up to Tbps | Long (>km) | Very High | Backbone of global telecommunications networks, long-distance communication, data centers |
| Twinax | 10-100 Gbps | Short (<10m) | High | Servers, switches, storage networks |

# 7. Serializer / Deserializer (SerDes)

- A serializer converts parallel data into a serial stream.

- A deserializer converts the serial stream back into parallel data.

SerDes enables high-speed transmission over fewer wires, reducing pin count and EMI (electromagnetic interference) while maintaining high data integrity.

## 7.1. Need for SerDes in video PROCESSING

Modern video applications require high resolution data and high frame rates.

Transmitting this data in parallel would require many wires and high power. SerDes helps in speed transmission by:

- Reducing the number of physical lines by serializing data

- Increasing bandwidth over a single or few lines

- Improving EMI/EMC characteristics

- Enabling long-distance transmission

## 7.2. WORKING OF SERDES

- High-speed digital video data is generated in parallel format by a video source

- The Serializer takes the parallel data lines and converts them into a single high-speed serial stream.

- This reduces the number of wires or PCB traces needed, enabling compact and low-noise transmission.

- The serial data is transmitted over a physical medium Since the signal is serialized, it travels faster and is less prone to interference or skew.

- The Deserializer receives the serial stream and converts it back into parallel data.

- It realigns and reconstructs the original multi-bit video data for further processing or display.

- The Deserializer also performs clock data recovery (CDR).

- It extracts the timing information from the serial stream to accurately reconstruct the original parallel video data.

- The system ensures the video data is synchronized, detecting frame boundaries so that the video frame can be properly displayed.

- The deserialized video is then sent to a display, video decoder, or processor for further rendering, decoding, or storage.

# 8. Electromagnetic Compatibility

Ability of an electronic system or device to function properly in its electromagnetic environment without:

- Emitting electromagnetic interference (EMI) that could disrupt other systems, and

- Being affected by EMI from other sources.

EMC:

- EMC (Electromagnetic Compatibility) is the ability of an electronic device/system to operate properly in its electromagnetic environment without:

- Emitting electromagnetic interference (EMI) that affects other systems.

- Being affected by EMI from other sources.

EMC = Minimal emission + Strong immunity.

EMC is engineered through:

- Design practices (shielding, grounding, filtering).

- PCB layout techniques.

- Compliance testing with EMC standards (like CISPR, FCC, etc.).

The two important things in EMC are:

- Emission suppression.

- Susceptibility hardening.

1. Emission Suppression

This is the technique used to reduce EMI from a device so that it meets EMC limits.

Techniques include:

- Shielding (metal enclosures to block radiation).

- Filtering (EMI filters on power lines).

- Twisted pair wiring, proper PCB grounding.

- Spread spectrum clocking to distribute EMI energy over a broader band.

- This addresses the "not interfering with others" part of EMC.

2. Susceptibility hardening.

This means making a device less sensitive to incoming EMI from the environment.

Techniques include:

- Input filtering.

- TVS diodes, common-mode chokes, and ESD protection.

- PCB layout that avoids long, unshielded traces.

- Isolation (opto-isolators, digital isolators).

- This addresses the "not being disturbed by others" part of EMC.

Note: A consumer device must meet regional EMC standards before it can be legally sold.

# 9. References

1.ch06.pptx

2.Things that affect video quality: Resolution, Color depth, Frame rate, Bitrate

3.What is TMDS and why is it in my HDMI

4.Things that affect video quality: Video compression

5.Introduction to Color Spaces in Video | Matrox Video