END APPLICATON WITH GMSL Setup

# Technical Specification & Installation Manual: Jetson Surveillance System

Document History:

| S. No | Ver.No. | Authors | Section(s) Affected | Change(s) made | Approved Date | Approved by |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.0 | Hemavathi A | All | Initial |  |  |

TABLE OF CONTENTS

Technical Specification & Installation Manual: Jetson Surveillance System	1

1. System Overview	3

2. Hardware Requirements	3

3. Environment Preparation	3

3.1 JetPack Setup	3

3.2 Global Dependencies	3

3.3 Create Virtual Environment	4

4. Software Installation	4

4.1 Required Python Packages	4

4.2 Install NVIDIA-Optimized PyTorch (Mandatory for CUDA)	4

4.3 Memory Optimization (Swap File)	4

5. CUDA & GPU Verification (Confirmation Steps)	5

5.1 Verify CUDA Compiler	5

5.2 Verify Python-GPU Link (The "Success" Test)	5

5.3 Monitor Real-Time GPU Load	5

6. AI Model Deployment (TensorRT)	5

6.1 Exporting Engines	6

7. Application Configuration	6

7.1 GStreamer Optimization (v4l2_driver.py)	6

7.2 Asset Zone Data (fence_config.json)	6

8. Execution Procedure	7

8.1 Hardware Initialization	7

8.2 Launching the System	7

9. Troubleshooting & Performance Tuning	7

## 1. System Overview

The application is designed for NVIDIA Jetson SoCs to provide real-time holographic surveillance.

- Cam 0: Face Detection & Identification.

- Cam 1 (HoloScan): Skeletal Pose Estimation & Face Wireframe mapping.

- Cam 2 (AssetZone): Circular Geofencing with automated intruder alerting.

## 2. Hardware Requirements

- SoC: NVIDIA Jetson (Nano, Xavier NX, or Orin Nano/NX).

- Cameras: 3x GMSL2 or CSI Cameras (1080p supported, 720p recommended for 3-cam stability).

- Power: 5V/4A (Nano) or 19V DC (Orin/Xavier) — Wall power is mandatory; USB power will cause crashes.

- Storage: 64GB+ High-speed microSD or NVMe SSD.

## 3. Environment Preparation

### 3.1 JetPack Setup

Ensure you are running JetPack 5.1.x or 6.x. Verify the L4T version:

cat /etc/nv_tegra_release

Before starting, ensure the Jetson OS knows where the CUDA libraries are located. Add these to your ~/.bashrc file:

export PATH=/usr/local/cuda/bin:$PATH
export LD_LIBRARY_PATH=/usr/local/cuda/lib64:$LD_LIBRARY_PATH
source ~/.bashrc

### 3.2 Global Dependencies

Install the system-level libraries required for OpenCV and GStreamer:

sudo apt-get update
sudo apt-get install -y libv4l-dev libv4l-0 python3-pip
sudo apt-get install -y libcanberra-gtk-module libcanberra-gtk3-module

### 3.3 Create Virtual Environment

To avoid breaking system paths:

python3 -m venv venv
source venv/bin/activate
pip install --upgrade pip

## 4. Software Installation

### 4.1 Required Python Packages

Install the core AI and GUI stack. Note: We use a specific NumPy version to prevent Scipy compatibility crashes.

pip install "numpy<1.24"
pip install opencv-python ultralytics PyQt5 polars ultralytics-thop
pip install scipy pillow requests pandas

### 4.2 Install NVIDIA-Optimized PyTorch (Mandatory for CUDA)

Important: Do NOT use pip install torch. You must use the Jetson-specific version to enable the GPU:

# Example for JetPack 5.1 (Adjust URL for your specific JetPack version)
pip install --no-cache-dir https://developer.download.nvidia.com/compute/redist/jp/v511/pytorch/torch-2.0.0+nv23.05-cp38-cp38-linux_aarch64.whl

### 4.3 Memory Optimization (Swap File)

Since Jetson has shared RAM/GPU memory, a swap file is mandatory to prevent the "Killed" (OOM) error.

sudo swapoff /swapfile || true
sudo fallocate -l 8G /swapfile
sudo chmod 600 /swapfile
sudo mkswap /swapfile
sudo swapon /swapfile
# Verify
free –h

## 5. CUDA & GPU Verification (Confirmation Steps)

Run these checks to confirm the installation is successful before launching the surveillance app.

### 5.1 Verify CUDA Compiler

nvcc --version

Expected: Must show "Cuda compilation tools, release XX.X".

### 5.2 Verify Python-GPU Link (The "Success" Test)

Run this inside your virtual environment:

python3 -c "import torch; print('CUDA Available:', torch.cuda.is_available()); print('GPU Device:', torch.cuda.get_device_name(0))"

- If True: The AI will run on the GPU (Fast).

- If False: The AI will run on the CPU (Application will freeze).

### 5.3 Monitor Real-Time GPU Load

While the app is running, open a second terminal and run:

tegrastats

Watch the GR3D_FREQ value. If it is above 0%, your CUDA kernels are active.

## 6. AI Model Deployment (TensorRT)

For the system to run without "Stuck Frames," you must use TensorRT .engine files instead of .pt files.

### 6.1 Exporting Engines

Run these commands once on the Jetson to generate the optimized hardware engines:

# Export Pose Engine for HoloScan (Cam 1)
yolo export model=yolov8n-pose.pt format=engine device=0 imgsz=320 half=True

# Export Detection Engine for AssetZone (Cam 0 & 2)
yolo export model=yolo11n.pt format=engine device=0 imgsz=320 half=True

## 7. Application Configuration

### 7.1 GStreamer Optimization (v4l2_driver.py)

Use max-buffers=1 to prevent the dmabuf -1 hardware lock when running 3 cameras at 1080p.

codePython

gst_str = (
    f"nvarguscamerasrc sensor-id={self.sensor_id} ! "
    f"video/x-raw(memory:NVMM), width=1920, height=1080, framerate=20/1 ! "
    f"nvvidconv ! video/x-raw, format=BGRx ! "
    f"videoconvert ! video/x-raw, format=BGR ! "
    f"appsink drop=True max-buffers=1"
)

### 7.2 Asset Zone Data (fence_config.json)

The application saves coordinates here. Ensure it is writable:

JSON

{
    "enabled": true,
    "cx": 960,
    "cy": 540,
    "radius": 200,
    "show_grid": true
}

## 8. Execution Procedure

### 8.1 Hardware Initialization

Always run the hardware reset before launching the app to clear the ISP buffers:

sudo systemctl restart nvargus-daemon
sudo nvpmodel -m 0
sudo jetson_clocks

### 8.2 Launching the System

source venv/bin/activate
python3 main.py

## 9. Troubleshooting & Performance Tuning

| Issue | Cause | Fix |
| --- | --- | --- |
| Killed | Out of Memory | Ensure 8GB Swap is active. |
| dmabuf -1 error | ISP Buffer Overflow | Reduce resolution to 720p and FPS to 21. |
| Stuck Frame | AI Thread Deadlock | Use .engine models and 320px imgsz. |
| Inaccurate Detection | Domain Gap | Fine-tune model with 1000 local snapshots. |
| CPU Overheat | Max Clock Heat | Ensure Jetson has active cooling (Fan). |